Broadcast (Terrestrial) Radio Interface Specifications

Contact Information

Spektrum Yönetimi Dairesi Başkanlığı

Bilgi Teknolojileri ve İletişim Kurumu

Eskişehir Yolu 10. km. No:276 Çankaya / Ankara

Tel		: +90 312 294 7310

E-posta	: MFP@btk.gov.tr

Web		: www.btk.gov.tr

List of Broadcast (Terrestrial) Radio Interface Specifications

| Reference Number | Service | Application | Frequency Band | Comment |
| --- | --- | --- | --- | --- |
| TAD07-01-01 | Broadcast | AM radio analog | 148.5-283.5 kHz | Long Wave |
| TAD07-01-02 | Broadcast | AM radio analog | 526.5 - 1606.5 kHz | Medium Wave |
| TAD07-01-03 | Broadcast | FM radio analog | 87.5 - 108.0 MHz | Band II |
| TAD07-01-04 | Broadcast | T-DAB | 216 - 230 MHz | Band III (Channel 11,12) |
| TAD07-01-05 | Broadcast | DVB-T2 | 174 - 216 MHz | Band III (Channel 5 - 10); 7 MHz |
| TAD07-01-06 | Broadcast | DVB-T2 | 470 - 694 MHz | Band IV / V (Channel 21 - 48); 8 MHz |

| Republic of Turkey Information and Communications Technologies Authority | Radio Interface Specification | AM radio analog | TAD07-01-01 |  |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Broadcast |  |
|  | 2 | Application | AM Radio analog |  |
|  | 3 | Frequency band | 148.5-283.5 kHz |  |
|  | 4 | Channelling | 9 kHz |  |
|  | 5 | Modulation / Occupied bandwidth | A3E |  |
|  | 6 | Direction / Separation | - |  |
|  | 7 | Transmit power / Power density | - |  |
|  | 8 | Channel access and occupation rules | - |  |
|  | 9 | Authorisation regime | - | From this band, broadcasting is made only by public broadcaster. |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions | EN 302 017-1 | It is subject to ITU/R Geneva 1975, GE75 Agreement. |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 302 017-2, EN 301 489-1, EN 301 489-11 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | - |  |

| Republic of Turkey Information and Communications Technologies Authority | Radio Interface Specification | AM radio analog | TAD07-01-02 |  |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Broadcast |  |
|  | 2 | Application | AM Radio analog |  |
|  | 3 | Frequency band | 526.5-1606.5 kHz |  |
|  | 4 | Channelling | 9 kHz |  |
|  | 5 | Modulation / Occupied bandwidth | A3E |  |
|  | 6 | Direction / Separation | - |  |
|  | 7 | Transmit power / Power density | - |  |
|  | 8 | Channel access and occupation rules | - |  |
|  | 9 | Authorisation regime | - | From this band, broadcasting is made only by public broadcaster. |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions | EN 302 017-1 | It is subject to ITU/R Geneva 1975, GE75 Agreement. |
| Informative Part | 12 | Planned changes | - |  |
|  | 13 | Reference | EN 302 017-2, EN 301 489-1, EN 301 489-11 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | - |  |

| Republic of Turkey Information and Communications Technologies Authority | Radio Interface Specification | FM radio analog | TAD07-01-03 |  |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Broadcast |  |
|  | 2 | Application | FM Radio analog |  |
|  | 3 | Frequency band | 87.5-108 MHz |  |
|  | 4 | Channelling | 100 kHz |  |
|  | 5 | Modulation / Occupied bandwidth | F3E | Maximum frequency deviation: +/- 75 kHz Pre-emphasis: 50 micro second |
|  | 6 | Direction / Separation | - | - |
|  | 7 | Transmit power / Power density |  | Power limits of transmitter are determined by RTÜK. |
|  | 8 | Channel access and occupation rules | - | - |
|  | 9 | Authorisation regime | Subject to permission by Radio TV Supreme Council (RTÜK) |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions | EN 302 018-1 | It is subject to ITU/R Geneva 1984, GE84 Agreement. |
| Informative Part | 12 | Planned changes | - | - |
|  | 13 | Reference | EN 302 018-2, EN 301 489-1, EN 301 489-11 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | - | - |

| Republic of Turkey Information and Communications Technologies Authority | Radio Interface Specification | T-DAB | TAD07-01-04 |  |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Broadcast |  |
|  | 2 | Application | T-DAB | Terrestrial Digital Audio Broadcasting (Band III) |
|  | 3 | Frequency band | 216-230 MHz | - |
|  | 4 | Channelling | 1.536 MHz | - |
|  | 5 | Modulation / Occupied bandwidth | QPSK, 16 QAM, 64 QAM | COFDMA (Coded Frequency Division Multiplex Access) |
|  | 6 | Direction / Separation | - |  |
|  | 7 | Transmit power / Power density | - |  |
|  | 8 | Channel access and occupation rules | - |  |
|  | 9 | Authorisation regime | Subject to permission by Radio TV Supreme Council (RTÜK) | T-DAB frequency planning is made by RTÜK. |
|  | 10 | Additional essential requirements | - | - |
|  | 11 | Frequency planning assumptions | EN 302 077-1 | It is subject to ITU/R Geneva 2006, Final Acts of Regional Radiocommunication Conference (RRC-06). |
| Informative Part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 609-4, ERC/DEC/(94)01, ERC/DEC/(97)02, ECC/REC/(05)08, EN 301 489-1, EN 301 489-11 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| Republic of Turkey Information and Communications Technologies Authority | Radio Interface Specification | DVB-T2 | TAD07-01-05 |  |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Broadcast | - |
|  | 2 | Application | DVB-T2 | - |
|  | 3 | Frequency band | 174 - 216 MHz | Band III |
|  | 4 | Channelling | 7 MHz | Channel 5-10 |
|  | 5 | Modulation / Occupied bandwidth | - | - |
|  | 6 | Direction / Separation | - | - |
|  | 7 | Transmit power / Power density | Class of Power 1: 5W-12.5 W Class of Power 2: 20W-100 W Class of Power 3: >100 W | RTÜK Turkish Terrestrial Digital Television (DVB-T2) Frequency Plan |
|  | 8 | Channel access and occupation rules | - | - |
|  | 9 | Authorisation regime | Subject to permission by Radio TV Supreme Council(RTÜK) | - |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions | EN 302 296 | It is subject to ITU/R Geneva 2006, Final Acts of Regional Radiocommunication Conference (RRC-06). |
| Informative Part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 302 296, EN 301 489-1, EN 301 489-14 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | - | - |

| Republic of Turkey Information and Communications Technologies Authority | Radio Interface Specification | DVB-T2 | TAD07-01-06 |  |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Broadcast |  |
|  | 2 | Application | DVB-T2 | - |
|  | 3 | Frequency band | 470 - 582 MHz 582 - 694 MHz | - |
|  | 4 | Channelling | 8 MHz | TV Channels 21-34 and 35-48 |
|  | 5 | Modulation / Occupied bandwidth | - | - |
|  | 6 | Direction / Separation | - | - |
|  | 7 | Transmit power / Power density | Class of Power 1: 5W-12.5 W Class of Power 2: 20W-100 W Class of Power 3: >100 W | RTÜK Turkish Terrestrial Digital Television (DVB-T2) Frequency Plan |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime | Subject to permission by Radio TV Supreme Council(RTÜK) | - |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions | EN 302 296 | It is subject to ITU/R Geneva 2006, Final Acts of Regional Radiocommunication Conference (RRC-06). |
| Informative Part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 302 296, EN 301 489-1, EN 301 489-14 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | - | - |