Land Mobile (PMR/PAMR) Radio Interface Specifications

Contact Information

Spektrum Yönetimi Dairesi Başkanlığı

Bilgi Teknolojileri ve İletişim Kurumu

Eskişehir Yolu 10. km. No:276 Çankaya / Ankara

Tel		: +90 312 294 7310

E-posta	: MFP@btk.gov.tr

Web		: www.btk.gov.tr

List of Land Mobile (PMR/PAMR) Radio Interface Specifications

| Reference Number | Service | Application | Frequency Band | Comment |
| --- | --- | --- | --- | --- |
| TAD01-02-01 | Land Mobile | PMR/PAMR | 136-174 MHz | Analog PMR/PAMR |
| TAD01-02-02 | Land Mobile | PMR/PAMR | 136-174 MHz | Digital PMR/PAMR |
| TAD01-02-03 | Land Mobile | PMR/PAMR | 380-470 MHz | Analog PMR/PAMR |
| TAD01-02-04 | Land Mobile | PMR/PAMR | 380-470 MHz | Digital PMR/PAMR |

| Republic of Turkey Information and Communications Technologies Authority | Radio Interface Specification | PMR/PAMR | TAD01-02-01 |  |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Land Mobile |  |
|  | 2 | Application | PMR/PAMR | Analog PMR/PAMR simplex and duplex. If devices have both analog and digital features, analog interfaces are used in analog use and digital interfaces are used in digital use |
|  | 3 | Frequency band | 136-174 MHz | 156.5125-156.5375 MHz and and 156.7625-156.8375 MHz Maritime mobile (danger and safety) channels are excluded |
|  | 4 | Channelling | 6.25 kHz / 12.5 kHz / 25 kHz | Currently, 6.25 kHz channel range planning is not available |
|  | 5 | Modulation / Occupied bandwidth | F3E, G3E, F8E, F3D, F8D |  |
|  | 6 | Direction / Separation | 500 kHz, 5 MHz, 10 MHz |  |
|  | 7 | Transmit power / Power density |  | The maximum power limit varies according to the assignment. 5 W is the preferred power limit |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime | Subject to frequency assignment and authorisation | Without prejudice to the Article 8/2 of Electronic Communication Law Numbered 5809 |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions | T/R 25-08, EN 300 113-1, EN 300 390-2, EN 301 166-1 |  |
| Informative Part | 12 | Planned changes |  |  |
|  | 13 | Reference | T/R 25-08, EN 301 489-1, EN 301 489-5, EN 303 039, EN 62368-1 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| Republic of Turkey Information and Communications Technologies Authority | Radio Interface Specification | PMR/PAMR | TAD01-02-02 |  |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Land Mobile |  |
|  | 2 | Application | PMR/PAMR | Digital PMR/PAMR simplex and duplex. If devices have both analog and digital features, analog interfaces are used in analog use and digital interfaces are used in digital use |
|  | 3 | Frequency band | 136-174 MHz | 156.5125-156.5375 MHz and and 156.7625-156.8375 MHz Maritime mobile (danger and safety) channels are excluded |
|  | 4 | Channelling | 6.25 kHz / 12.5 kHz / 25 kHz | Currently, 6.25 kHz channel range planning is not available |
|  | 5 | Modulation / Occupied bandwidth | F3E, G3E, F7D, F7E |  |
|  | 6 | Direction / Separation | 500 kHz, 5 MHz, 10 MHz |  |
|  | 7 | Transmit power / Power density |  | The maximum power limit varies according to the assignment. 5 W is the preferred power limit |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime | Subject to frequency assignment and authorisation | Without prejudice to the Article 8/2 of Electronic Communication Law Numbered 5809 |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions | T/R 25-08, TR 102 398, EN 300 113-1, EN 300 390-1, EN 301 166-1 |  |
| Informative Part | 12 | Planned changes |  |  |
|  | 13 | Reference | T/R 25-08, TS 102 361, TS 100 392, EN 301 489-1, EN 301 489-5, EN 303 405,EN 300 392-9, EN 300 396-6, EN 303 039, EN 300 390, EN 301 166, EN 300 113, EN 62368-1 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| Republic of Turkey Information and Communications Technologies Authority | Radio Interface Specification | PMR/PAMR | TAD01-02-03 |  |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Land Mobile |  |
|  | 2 | Application | PMR/PAMR | Analog PMR/PAMR simplex and duplex. If devices have both analog and digital features, analog interfaces are used in analog use and digital interfaces are used in digital use |
|  | 3 | Frequency band | 380-470 MHz | 452-457 MHz/462-467 MHz frequency bands excluded |
|  | 4 | Channelling | 6.25 kHz, 12.5 kHz, 25 kHz | Currently, 6.25 kHz channel range planning is not available |
|  | 5 | Modulation / Occupied bandwidth | F3E, G3E, F8E, F3D, F8D, |  |
|  | 6 | Direction / Separation | 5 MHz, 10 MHz |  |
|  | 7 | Transmit power / Power density |  | The maximum power limit varies according to the assignment. 5 W is the preferred power limit |
|  | 8 | Channel access and occupation rules | No restriction |  |
|  | 9 | Authorisation regime | Subject to frequency assignment and authorisation | Without prejudice to the Article 8/2 of Electronic Communication Law Numbered 5809 |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions | T/R 25-08, EN 300 113-1, EN 300 390-2, EN 301 166-1 |  |
| Informative Part | 12 | Planned changes |  |  |
|  | 13 | Reference | T/R 25-08, EN 301 489-1, EN 301 489-5, EN 303 039, EN 62368-1 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| Republic of Turkey Information and Communications Technologies Authority | Radio Interface Specification | PMR/PAMR | TAD01-02-04 |  |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative Part | 1 | Radiocommunication Service | Land Mobile |  |
|  | 2 | Application | PMR/PAMR | Digital PMR/PAMR simplex and duplex. If devices have both analog and digital features, analog interfaces are used in analog use and digital interfaces are used in digital use |
|  | 3 | Frequency band | 380-470 MHz | 452-457 MHz/462-467 MHz frequency bands excluded |
|  | 4 | Channelling | 6.25 kHz, 12.5 kHz, 25 kHz | Currently, 6.25 kHz channel range planning is not available |
|  | 5 | Modulation / Occupied bandwidth | F3E, G3E, F7D, F7E |  |
|  | 6 | Direction / Separation | 5 MHz, 10 MHz |  |
|  | 7 | Transmit power / Power density |  | The maximum power limit varies according to the assignment. 5 W is the preferred power limit |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime | Subject to frequency assignment and authorisation | Without prejudice to the Article 8/2 of Electronic Communication Law Numbered 5809 |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions | T/R 25-08, TR 102 398, EN 300 113-1, EN 300 390-2, EN 301 166-1 |  |
| Informative Part | 12 | Planned changes |  |  |
|  | 13 | Reference | T/R 25-08, TS 102 361, TS 100 392, EN 301 489-1, EN 301 489-5, EN 300 392-9, EN 300 396-6, EN 303 039, EN 300 390, EN 301 166, EN 300 113, EN 62368-1 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |